There are two ways to replace the DMI as below:

- Modify in Internal shell.

ShellFlash64.efi /dms “Type 1 System Manufacture name” /dps “Type 1 System Product name” /dss “Serial Number” /dks “SKU number” /nodelay /patch /exit

DMI type 11, feature_byte & build_id , command:

winflash /dos 1 “FBYTE#feature_bytenumber;” 2 “BUILDID#buildnumber;” /nodelay /patch /exit

- Modify in Windows.

WinFlash64.exe /dss “Type 1 System Serial number” /nodelay /patch /exit

Note. The string will update after next reboot.

Usage: WinFlash64.exe [COMMAND]

dat      string           Specify the asset tag DMI string.

dmc      string           Specify the chassis manufacturer DMI string.

dmm      string           Specify the motherboard manufacturer DMI string.

dks      string           Specify the SKU number DMI string.

dms      string           Specify the system manufacturer DMI string.

dos      [string;string2;...]|[index1 string1 ...] Specify the OEM DMI strings.

dpc      string           Specify the chassis asset tag number DMI string.

dpm      string           Specify the motherboard product ID DMI string.

dps      string           Specify the system product ID DMI string.

dsc      string           Specify the chassis serial number DMI string.

dsm      string           Specify the motherboard serial number DMI string.

dss      string           Specify the system serial number DMI string.

dus      [uuid]           Specify the UUID DMI string.

dvc      string           Specify the chassis version DMI string.

dvm      string           Specify the motherboard version DMI string.

dvs      string           Specify the system version DMI string.